CIT602 COMPUTER ARCHITECTURE

NAMES:KATLEHO QHOBELA

STUDENT NUMBER:20250016

WEEK11 SELF-STUDY

Option 1: I/O Interface Comparison

Chosen: USB 4.0 vs PCIe 5.0 vs SATA III

Characteristic	USB 4.0	PCIe 5.0 x16SATA III

Max Bandwidth	40 Gbps	63 GB/s / 504 Gbps	6 Gbps / 600 MB/s

Latency	Medium ~10 μs	Very Low <1 μs	Higher ~10-20 μs

Topology	External, daisy-chain/tree via hubs	Internal, point-to-point lanes	Internal, point-to-point

Hot Swappable	Yes	No, requires shutdown	Yes, with AHCI

Typical Applications	External SSDs, docks, displays, charging	GPUs, NVMe SSDs, network cards	HDDs, 2.5" SATA SSDs

Power Delivery	Up to 240W	75W from slot, more via cables	4.5W typical

Analysis:

PCIe 5.0 dominates for internal high-performance needs like GPUs and NVMe drives due to massive bandwidth and ultra-low latency. The point-to-point topology gives each device dedicated lanes. SATA III is legacy tech now, limited to HDDs and older SSDs because 600 MB/s bottlenecks modern NAND. USB 4.0 is the king of external connectivity, balancing 40 Gbps speed with power delivery and universal compatibility. Use PCIe for performance, SATA for cheap bulk storage, USB 4 for everything external.

Option 2: Transfer Method Analysis

1. Three Peripheral Devices

1. Keyboard

2. Gigabit Ethernet NIC

3. NVMe SSD

2. Optimal Transfer Method + 3. Justification

Device	Optimal Method	Justification: Speed, Data Size, CPU Impact

Keyboard	Interrupt-driven I/O	Speed: Very slow, ~10-100 bytes/sec. Data size: Tiny, 1 byte per key. CPU impact: Programmed I/O wastes CPU polling. Interrupts only fire when key pressed, so CPU is free 99.9% of time.

Gigabit NIC	DMA	Speed: High, 125 MB/s. Data size: Large packets, 1500 bytes each, continuous. CPU impact: Interrupt per packet would overload CPU. DMA moves packets straight to RAM, CPU only handles completion interrupt. Critical for network throughput.

NVMe SSD	DMA	Speed: Extreme, 7+ GB/s. Data size: Huge, 4KB-128KB blocks, queued. CPU impact: Programmed I/O impossible, would stall entire system. DMA + NVMe queues allow 64K commands with minimal CPU usage.

Report Summary: Use interrupt-driven for slow, unpredictable devices like keyboards/mice to save CPU cycles. Use DMA for high-speed, block-oriented devices like NICs and SSDs where CPU involvement would be a bottleneck. Programmed I/O is only viable for simple devices during system boot before interrupts are enabled.

Option 3: I/O Performance Calculation

Scenario: Transfer 64 KB from disk. Disk data rate = 100 MB/s. CPU processes 1 instruction per 2 ns. Programmed I/O: 4 instructions/word. Interrupt: 1000 instructions overhead per transfer. DMA: 2000 instructions setup. 1 word = 4 bytes.

1. Transfer size: 64 KB = 65536 bytes = 16384 words

2. Calculate transfer times:

Method	CPU Instructions	CPU Time	Transfer Time	CPU Utilization

Programmed I/O	16384 words × 4 = 65536 instructions	65536 × 2ns = 131 μs	Disk time = 65536 B / 100 MB/s = 655 μs. CPU busy for all 655 μs waiting	100% during transfer

Interrupt-driven	16384 interrupts × 1000 = 16.4M instructions	16.4M × 2ns = 32.8 ms	Disk time = 655 μs. But CPU spends 32.8 ms on overhead	32800 / 655 = 5000% - impossible, CPU is bottleneck

DMA	Setup = 2000 instructions	2000 × 2ns = 4 μs	Disk time = 655 μs. CPU free after 4 μs setup	4 / 655 = 0.6%

3. Analysis of System Impact:

- Programmed I/O: CPU at 100% utilization and completely blocked for 655 μs. System freezes. Only usable for tiny transfers.

- Interrupt-driven: Interrupt overhead exceeds transfer time by 50x. CPU becomes the bottleneck, making disk appear slower. Terrible for block devices.

- DMA: CPU utilization 0.6%. Disk runs at full 100 MB/s while CPU handles other tasks. This is why all modern disks use DMA. The 4 μs setup cost is negligible.

Interpretation: For large block transfers, DMA is the only viable method. It reduces CPU load from 100% to <1% and allows true parallel operation between CPU and I/O. Interrupt-driven only makes sense for small, infrequent transfers.